1.	PRODUCT AND COMPANY IDENTIFICATION:

PRODUCT: Gavel* 80 WDG Fungicide

COMPANY IDENTIFICATION:

Dow AgroSciences LLC

9330 Zionsville Road

Indianapolis, IN 46268-1189

2.	COMPOSITION/INFORMATION ON INGREDIENTS:

Mancozeb:                       CAS # 008018-01-7            66.7%

[[1,2-ethanediylbis-

[carbamodithioato]](2-)]

manganese mixture with

[[1,2-ethanediylbis

[carbamodithioato]](2-)]zinc

Zoxamide:                       CAS # 156052-68-5              8.3%

3,5-dichloro-N-(3-chloro-

1-ethyl-1-methyl-2-oxopropyl)-

4-methylbenzamide

Balance, Total, Including:                                           25.0%

Calcium Lignosulfonate        CAS # 008061-52-7

Ethylene Thiourea                CAS # 000096-45-7

Hexamethylene Tetramine   CAS # 000100-97-0

3.	HAZARDOUS IDENTIFICATIONS:

EMERGENCY OVERVIEW

Hazardous chemical. Brown granule with a sulfur-like odor. May cause eye irritation with corneal injury. May cause skin irritation. LD50 for skin absorption and oral ingestion is >5000 mg/kg. Inhalation LC50 is >5.0 mg/L for 4 hours.

EMERGENCY PHONE NUMBER: 800-992-5994

POTENTIAL HEALTH EFFECTS: This section includes possible adverse effects, which could occur if this material is not handled in the recommended manner.

EYE: May cause moderate eye irritation with corneal injury.

SKIN: Brief contact may cause slight skin irritation with local redness. Prolonged skin contact is unlikely to result in absorption of harmful amounts. Prolonged or frequently repeated skin contact may cause allergic skin reactions in some individuals. The LD50 for skin absorption in rats for a similar material is >5000 mg/kg.

INGESTION: Very low toxicity if swallowed. Harmful effects not anticipated from swallowing small amounts. Based on information for a similar material, the oral LD50 for rats is expected to be >5000 mg/kg.

INHALATION: Dust may cause irritation of the upper respiratory tract (nose and throat) and lungs. The expected LC50 for rats is >5.0 mg/L for 4 hours.

SYSTEMIC (OTHER TARGET ORGAN) EFFECTS: For the active ingredients, in animals, effects have been reported on the following organs: thyroid, liver, eye and nervous system.

CANCER INFORMATION: Mancozeb has caused cancer in laboratory animals. Zoxamide did not cause cancer in laboratory animals.

TERATOLOGY (BIRTH DEFECTS): Mancozeb has caused birth defects in laboratory animals only at doses toxic to the mother. Zoxamide did not cause birth defects or any other fetal effects in laboratory animals.

REPRODUCTIVE EFFECTS: The active ingredients, in animal studies, did not interfere with reproduction.

4.	FIRST AID:

EYE: Hold eyes open and rinse slowly and gently with water for 15-20 minutes. Remove contact lenses, if present, after the first 5 minutes, then continue rinsing eyes. Call a poison control center or doctor for treatment advice.

SKIN: Take off contaminated clothing. Wash skin with soap and plenty of water for 15-20 minutes. Call a poison control center or doctor for treatment advice. Wash clothing before reuse. Discard shoes and other leather items, which cannot be decontaminated.

INGESTION: Call a poison control center or doctor immediately for treatment advice. Have person sip a glass of water if able to swallow. Do not induce vomiting unless told to do so by the poison control center or doctor. Never give anything by mouth to an unconscious person.

INHALATION: Move person to fresh air; if effects occur, consult a physician.

NOTE TO PHYSICIAN: No specific antidote. Treatment of exposure should be directed at the control of symptoms and the clinical condition of the patient.

5.	FIRE FIGHTING MEASURES:

FLASH POINT: Not applicable

METHOD USED: Not applicable

FLAMMABLE LIMITS:

LFL: Not applicable

UFL: Not applicable

EXTINGUISHING MEDIA: Use the following extinguishing media when fighting fires involving this material: carbon dioxide, dry chemical, water spray or foam.

FIRE AND EXPLOSION HAZARDS: Pesticide particulates can become airborne. Dusts at sufficient concentrations can form explosive mixtures with air. Combustion generates toxic fumes of the following: nitrogen oxides, hydrogen chloride, hydrogen sulfide, sulfur oxides, and carbon oxides.

FIRE-FIGHTING EQUIPMENT: Wear self-contained breathing apparatus (pressure-demand NIOSH approved or equivalent) and full protective equipment.

SPECIAL PROCEDURES: Remain upwind. Avoid breathing smoke. Use water spray to cool containers exposed to fire. Contain run-off.

6.	ACCIDENTAL RELEASE MEASURES:

ACCIDENTAL RELEASE MEASURES: Sweep up small spills and transfer material to suitable containers for recovery or disposal. Keep dust to a minimum. Keep spectators away. Avoid breathing dust. Keep spills and cleaning runoff out of municipal sewers and open bodies of water. Report large spills to Dow AgroSciences at 800-992-5994. If exposed to material during cleanup, remove all contaminated clothing promptly. Wash all exposed skin areas with soap and water immediately after exposure. Thoroughly launder clothing before reuse. Do not take clothing home to be laundered.

7.	HANDLING AND STORAGE:

HANDLING AND STORAGE:

HANDLING: Do not handle material near food, feed or drinking water. This material is a potential skin sensitizer. Avoid high concentrations of dust in air and accumulation of dust on equipment. An airborne dust of this material can create a dust explosion. When handling and processing this material local exhaust ventilation may be required to control and reduce exposure to vapors. To prevent dust explosions employ bonding and grounding for operations capable of generating static electricity. Protect all equipment from explosions by following the guidelines in NFPA-68 and NFPA-69. For electrical equipment follow local codes and electrical classification NFPA-70 (the National Electrical Code), class II, division 2, group G.

STORAGE: Do not store this material near food, feed or drinking water. Keep container tightly closed when not in use. Store in a dry area. Store in a well-ventilated area. DO NOT allow material to become wet or over-heated in storage; decomposition, impaired activity or fire may result. Material is combustible; do not ignite. Store bagged material only on pallets no more than 3 high. Provide access aisles for each 2 rows. Loose bags should not be stacked more than 2x2x2 meters. Dense packing of unvented stacks of bags may lead to product decomposition posing a fire hazard. Decomposition produces a foul odor. Check for hot containers and immediately remove to open areas for disposal.

OTHER: Completely empty bag into application equipment. Dispose of empty bag in a sanitary landfill as allowed by state and local authorities.

8.	EXPOSURE CONTROLS/PERSONAL PROTECTION:

These precautions are suggested for conditions where the potential for exposure exists. Emergency conditions may require additional precautions.

EXPOSURE GUIDELINES:

Zoxamide: Dow AgroSciences Industrial Hygiene Guide is 3 mg/M3 (respirable) TWA.

Mancozeb: Dow AgroSciences Industrial Hygiene Guide is 1 mg/M3, TWA

ENGINEERING CONTROLS: Use only with adequate ventilation. Local exhaust ventilation may be necessary for some operations.

RECOMMENDATIONS FOR MANUFACTURING, COMMERCIAL BLENDING, AND PACKAGING

WORKERS:

EYE/FACE PROTECTION: Use chemical goggles.

SKIN PROTECTION: Use gloves chemically resistant to this material. When prolonged or frequently repeated contact could occur, use chemically protective clothing resistant to this material. Selection of specific items such as face shield, gloves, boots, apron, or full-body suit will depend on operation.

RESPIRATORY PROTECTION: In dusty or misty atmospheres, use a NIOSH approved particulate respirator.

APPLICATORS AND ALL OTHER HANDLERS: Refer to the product label for personal protective clothing and equipment.

9.	PHYSICAL AND CHEMICAL PROPERTIES:

COLOR: Brown

APPEARANCE: Granule

ODOR: Sulfur-like

pH: 6.5 to 7.5

VISCOSITY: Not applicable

SPECIFIC GRAVITY (WATER = 1): 0.5 to 0.75 g/mL

(bulk density)

VAPOR DENSITY (AIR = 1): Not applicable

VAPOR PRESSURE: Negligible

MELTING POINT: Not applicable (decomposes)

BOILING POINT: Not applicable

SOLUBILITY IN WATER: Dispersible

PERCENT VOLATILITY: <3% (water)

EVAPORATION RATE (BAc = 1): Not applicable

10.	STABILITY AND REACTIVITY:

STABILITY: (CONDITIONS TO AVOID) Stable under normal storage conditions. Keep away from moisture, heat or flame.

INCOMPATIBLITY: (SPECIFIC MATERIALS TO AVOID) Avoid contact with acids and oxidizing agents.

HAZARDOUS DECOMPOSITION PRODUCTS: Thermal decomposition may yield the following: hydrogen chloride, carbon disulfide, and hydrogen sulfide.

HAZARDOUS POLYMERIZATION: Not known to occur.

11.	TOXICOLOGICAL INFORMATION:

MUTAGENICITY: For mancozeb, in-vitro genetic toxicity studies were predominantly negative and animal genetic toxicity studies were negative. For zoxamide, in-vitro genetic toxicity studies were negative.

12.	ECOLOGICAL INFORMATION:

ENVIRONMENTAL FATE:

DEGRADATION & PERSISTENCE:

Based on information for Zoxamide:

Octanol water partition (Kow) = 5782

Bioconcentration factor (BCF) = 95 - 136

Biodegradation:

Aerobic soil half-life = 2 - 10 days

Hydrolysis half-life = 15.7 days (pH 7)

Water sediment half-life = 6.1 days

Koc = 815 - 1671

Soil mobility is low

Based on information for Mancozeb:

Octanol water partition (Kow) = 21

Biodegradation:

Aerobic soil half-life = 2 hrs.

Hydrolysis half-life = 5.8 - 55 hrs. pH 7)

Water sediment half-life = 10 - 21 hrs.

Koc = 363 - 20,726

Soil mobility is medium to immobile.

ECOTOXICOLOGY:

Based on information for this material:

Honeybee oral LD50 is >153 µg/bee

Honeybee contact LD50 is >200 µg/bee

Algae (Selenastrum capricornutum) 96 hr EC50 is 31 µg/L

Daphnia magna 48 hr. EC50 is 3.3 mg/L

Rainbow trout (Oncorhynchus mykiss) 96 hr. LC50 is 1.9 mg/L

Based on information for zoxamide:

Rainbow trout (Oncorhynchus mykiss), 96 hour LC50 = 0.16 mg/L

Bluegill sunfish (Lepomis macrochirus), 96 hour LC50 = >0.79 mg/L

Daphnia magna 48 hour EC50 is >0.78 mg/L

Algae (Selenastrum capricornutum), 120 hour EC50 is 19 µg/L

Bobwhite quail LD50 is >2000 mg/kg

Mallard duck, 5 day LC50 is >5250 ppm

Honeybee contact LC50 is >100 µg/bee

Based on information for mancozeb:

Mallard duck 10 day LD50 is >6400 mg/kg.

13.	DISPOSAL CONSIDERATIONS:

DISPOSAL METHOD: Do not contaminate water, food or feed by storage or disposal. Excess wastes resulting from the use of this product may be disposed of on site according to label directions or at an approved waste disposal facility. Follow all local, state, and federal requirements for disposal.

14.	TRANSPORT INFORMATION:

U.S. DEPARTMENT OF TRANSPORTATION (D0T) INFORMATION:

For domestic non-bulk shipments:

This material is not regulated for transport.

For bulk and vessel shipments:

ENVIRONMENTALLY HAZARDOUS SUBSTANCES,

SOLID, N.O.S. (MANCOZEB)/9/UN3077/PGIII/

MARINE POLLUTANT (MANCOZEB)

15.	REGULATORY INFORMATION:

NOTICE:  The information herein is presented in good faith and believed to be accurate as of the effective date shown above.  However, no warranty, express or implied, is given.  Regulatory requirements are subject to change and may differ from one location to another; it is the buyer’s responsibility to ensure that its activities comply with federal, state or provincial, and local laws.  The following specific information is made for the purpose of complying with numerous federal, state or provincial, and local laws and regulations.

U.S. REGULATIONS

SARA 313 INFORMATION: This product contains the following substances subject to the reporting requirements of Section 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 and 40 CFR Part 372:

CHEMICAL NAME   CAS NUMBER    CONCENTRATION

Ethylene Thiourea      000096-45-7                 <0.1%

Mancozeb                   008018-01-7                 66.7%

Generic Group Name for Mancozeb:

Ethylenebisdithiocarbamic Acid (CAS # 000111-54-6)

SARA HAZARD CATEGORY: This product has been reviewed according to the EPA "Hazard Categories" promulgated under Sections 311 and 312 of the Superfund Amendment and Reauthorization Act of 1986 (SARA Title III) and is considered, under applicable definitions, to meet the following categories:

An immediate health hazard

A delayed health hazard

CALIFORNIA PROPOSITION 65: The following statement is made in order to comply with the California Safe Drinking Water and Toxic Enforcement Act of 1986:

This product contains trace levels of a component known to the state of California to cause cancer, birth defects, or other reproductive harm. The component is Ethylene thiourea (000096-45-7)

This product contains a component know to the state of California to cause cancer. The component is Mancozeb (008018-01-7)

TOXIC SUBSTANCES CONTROL ACT (TSCA): All ingredients are on the TSCA inventory or are not required to be listed on the TSCA inventory.

STATE RIGHT-TO-KNOW: The following product components are cited on certain state lists as mentioned. Non-listed components may be shown in the composition section of the MSDS.

# CHEMICAL NAME          CAS NUMBER       LIST

Ethylene Thiourea              000096-45-7    NJ1 NJ2 NJ3

PA1 PA2 PA3

Hexamethylene Tetramine 000100-97-0           NJ3

Ethylenebisdithiocarbamic  000111-54-6   NJ3 PA1 PA3

Acid

NJ1=New Jersey Special Health Hazard Substance (present at greater than or equal to 0.1%).

NJ2=New Jersey Environmental Hazardous Substance (present at greater than or equal to 1.0%).

NJ3=New Jersey Workplace Hazardous Substance (present at greater than or equal to 1.0%).

PA1=Pennsylvania Hazardous Substance (present at greater than or equal to 1.0%).

PA2=Pennsylvania Special Hazardous Substance (present at greater than or equal to 0.01%).

PA3=Pennsylvania Environmental Hazardous Substance (present at greater than or equal to 1.0%).

OSHA HAZARD COMMUNICATION STANDARD: This product is a "Hazardous Chemical" as defined by the OSHA Hazard Communication Standard, 29 CFR 1910.1200.

NATIONAL FIRE PROTECTION ASSOCIATION (NFPA) RATINGS:

Health                2

Flammability      1

Reactivity           1

COMPREHENSIVE ENVIRONMENTAL RESPONSE COMPENSATION AND LIABILITY ACT (CERCLA, or SUPERFUND): This product contains the following substance(s) listed as "Hazardous Substances" under CERCLA, which may require reporting of releases:

# Chemical Name          CAS#          RQ     % in Product

Ethylene Thiourea   000096-45-7    10 lbs.      <0.1%

Mancozeb               008018-01-7  5000 lbs.     66.7%

Generic Group Name for Mancozeb:

Ethylenebisdithiocarbamic Acid (CAS # 000111-54-6)

RCRA Categorization:

U.S. EPA Hazardous Waste Number:

U114 (Ethylenebisdithiocarbamic Acid)

16.	OTHER INFORMATION:

MSDS STATUS: Move to DAS format

Reference: DR-0383-5550

Replaces MSDS Dated: 6/1/01

The Information Herein Is Given In Good Faith, But No Warranty, Express or Implied, Is Made. Consult Dow AgroSciences for Further Information.