1.	PRODUCT AND COMPANY IDENTIFICATION:

PRODUCT: Rainbow* Herbicide

COMPANY IDENTIFICATION:

PT. DOW AGROSCIENCES INDONESIA

WISMA GKBI, FL. 20 - SUITE 2001

JL.JENDRAL SUDIRMAN NO. 28

JAKARTA.

TELP. NO. (021) 5742020 (Jam Kerja)

2.	COMPOSITION/INFORMATION ON INGREDIENTS:

Penoxsulam: 2-(2,2-           CAS # 219714-96-2          2.7%

difluoroethoxy)-N-(5,8-

dimethoxy[1,2,4]triazolo

[1,5-c]pyrimidin-2-yl)-6-

(trifluoromethyl)benzenesulfonamide

Other Ingredients, Total                                             97.3%

This document is prepared pursuant to the OSHA Hazard Communication Standard (29 CFR 1910.1200).  In addition, other substances not ‘Hazardous’ per this OSHA Standard may be listed.  Where proprietary ingredient shows, the identity may be made available as provided in this standard.

3.	HAZARDOUS IDENTIFICATIONS:

EMERGENCY OVERVIEW

Yellow opaque liquid with an oily odor. May cause eye and skin irritation. LD50 for skin absorption is >5000 mg/kg. Oral LD50 is >5000 mg/kg. Aerosol LC50 is >2.1 mg/L for 4 hours.

EMERGENCY PHONE NUMBER: 800-992-5994

POTENTIAL HEALTH EFFECTS: This section includes possible adverse effects, which could occur if this material is not handled in the recommended manner.

EYE: May cause moderate eye irritation. Corneal injury is unlikely.

SKIN: Prolonged contact may cause slight skin irritation with local redness. May cause drying and flaking of the skin. Prolonged skin contact is unlikely to result in absorption of harmful amounts. The LD50 for skin absorption in rats is >5000 mg/kg. Did not cause allergic skin reactions when tested in guinea pigs.

INGESTION: Very low toxicity if swallowed. Harmful effects not anticipated from swallowing small amounts. The oral LD50 for rats is >5000 mg/kg.

INHALATION: Brief exposure (minutes) is not likely to cause adverse effects. The aerosol LC50 for rats is >2.1 mg/L for 4 hours. This was the highest attainable concentration.

SYSTEMIC (OTHER TARGET ORGAN) EFFECTS: For penoxsulam, in animals, effects have been reported on the following organs: kidney and liver. No relevant information found on the other components.

CANCER INFORMATION: Penoxsulam did not cause cancer in laboratory animals.

TERATOLOGY (BIRTH DEFECTS): Penoxsulam did not cause birth defects or other effects in the fetus even at doses which caused toxic effects in the mother.

REPRODUCTIVE EFFECTS: In animal studies, penoxsulam has been shown not to interfere with reproduction.

4.	FIRST AID:

EYES: Flush eyes thoroughly with water for several minutes. Remove contact lenses after initial 1-2 minutes and continue flushing for several additional minutes. If effects occur, consult a physician, preferably an ophthalmologist.

SKIN: Wash skin with plenty of water.

INGESTION: No emergency medical treatment necessary.

INHALATION: Move person to fresh air; if effects occur, consult a physician.

NOTE TO PHYSICIAN: Skin contact may aggravate pre-existing dermatitis. No specific antidote. Treatment of exposure should be directed at the control of symptoms and the clinical condition of the patient.

5.	FIRE FIGHTING MEASURES:

FLASH POINT: >199oF (>93oC)

METHOD USED: SCC

# FLAMMABLE LIMITS

LFL: Not determined

UFL: Not determined

EXTINGUISHING MEDIA: Water, Foam or Dry Chemical.

FIRE AND EXPLOSION HAZARDS: This material does not burn, fight fire for other material that is burning.

FIRE-FIGHTING EQUIPMENT: Wear positive-pressure self-contained breathing apparatus (SCBA) and protective fire fighting clothing (includes fire-fighting helmet, coat, pants, boots and gloves).

6.	ACCIDENTAL RELEASE MEASURES:

ACTION TO TAKE FOR SPILLS: Absorb small spills with materials such as sand, sawdust, Zorball, or dirt. Wash exposed body areas thoroughly after handling. Report large spills to Dow AgroSciences at 800-992-5994.

7.	HANDLING AND STORAGE:

PRECAUTIONS TO BE TAKEN IN HANDLING AND STORAGE: Keep out of reach of children. Do not swallow. Avoid contact with eyes, skin, and clothing. Avoid breathing vapors or spray mist. Handle concentrate in ventilated area. Wash thoroughly with soap and water after handling and before eating, chewing gum, using tobacco, using the toilet or smoking. Keep away from food, feedstuffs, and water supplies. Store in original container with the lid tightly closed. Store in ventilated area.

8.	EXPOSURE CONTROLS/PERSONAL PROTECTION:

These precautions are suggested for conditions where the potential for exposure exists. Emergency conditions may require additional precautions.

EXPOSURE GUIDELINE(S): None established.

ENGINEERING CONTROLS: Good general ventilation should be sufficient for most conditions. Local exhaust ventilation may be necessary for some operations.

RECOMMENDATIONS FOR MANUFACTURING, COMMERCIAL BLENDING, AND PACKAGING WORKERS:

EYE/FACE PROTECTION: Use chemical goggles.

SKIN PROTECTION: Use gloves chemically resistant to this material when prolonged or frequently repeated contact could occur.

RESPIRATORY PROTECTION: For most conditions no respiratory protection should be needed; however, if discomfort is experienced, use a NIOSH approved air-purifying respirator.

APPLICATORS AND ALL OTHER HANDLERS: Refer to the product label for personal protective clothing and equipment.

9.	PHYSICAL AND CHEMICAL PROPERTIES:

APPEARANCE: Yellow opaque liquid

ODOR: Oily

DENSITY: 0.93 g/mL @ 20oC

pH: 4.96 @ 22.5oC

MELTING POINT: Not applicable

BOILING POINT: Not applicable

VAPOR PRESSURE: Not determined

SOLUBILITY: Not applicable

10.	STABILITY AND REACTIVITY:

STABILITY: (CONDITIONS TO AVOID) Stable under normal storage conditions.

INCOMPATIBILITY: (SPECIFIC MATERIALS TO AVOID) None known.

HAZARDOUS DECOMPOSITION PRODUCTS: None known.

HAZARDOUS POLYMERIZATION: Not known to occur.

11.	TOXICOLOGICAL INFORMATION:

MUTAGENICITY: For penoxsulam, in-vitro and animal mutagenicity studies were negative. No relevant information found on the other components.

12.	ECOLOGICAL INFORMATION:

ENVIRONMENTAL FATE:

MOVEMENT & PARTITIONING:

Based largely or completely on data for major component(s):

Bioconcentration potential is low (BCF is <100 or Low Pow >7).

Expected to be relatively immobile in soil (Koc is >5000).

DEGRADATION & PERSISTENCE:

Based largely or completely on information for major component(s):

Material is readily biodegradable. Passes OECD test(s) for ready biodegradability.

ECOTOXICOLOGY:

Material is practically non-toxic to aquatic organisms on an acute basis (LC50 or EC50 is >100 mg/L in the most sensitive species tested).

Acute LC50 in rainbow trout (Oncorhynchus mykiss) is >100 mg/L.

Acute immobilization EC50 in water flea (Daphnia magna) is 404 mg/L.

Acute oral LD50 in honeybee (Apis mellifera) is >170 µg/bee.

Acute oral LD50 in mallard (Anas platyrhynchos) is >2000 mg/kg.

Growth inhibition EC50 in green alga (Selenastrum capricornutum) is 4.0 mg/L.

Acute contact LD50 in honeybee (Apis mellifera) is >100 µg/bee.

The LC50 in earthworm (Eisenia foetida) is >10,000 mg/kg.

13.	DISPOSAL CONSIDERATIONS:

DISPOSAL METHOD: Do not contaminate water, food, or feed by storage or disposal. Excess wastes resulting from the use of this product may be disposed of on site according to label directions or at an approved waste disposal facility. Follow all local, state, and federal requirements for disposal.

14.	TRANSPORT INFORMATION:

This material will not form an aerosol under normal transportation conditions, therefore, the aerosol LC50 data will not apply when determining transportation information.

U.S. DEPARTMENT OF TRANSPORTATION (DOT) INFORMATION:

This product is not regulated by the U.S. DOT as a Hazardous Material.

15.	REGULATORY INFORMATION:

NOTICE:  The information herein is presented in good faith and believed to be accurate as of the effective date shown above.  However, no warranty, express or implied, is given.  Regulatory requirements are subject to change and may differ from one location to another; it is the buyer’s responsibility to ensure that its activities comply with federal, state or provincial, and local laws.  The following specific information is made for the purpose of complying with numerous federal, state or provincial, and local laws and regulations.

U.S. REGULATIONS

SARA 313 INFORMATION:  To the best of our knowledge, this product contains no chemical subject to SARA Title III Section 313 supplier notification requirements.

SARA HAZARD CATEGORY:  This product has been reviewed according to the EPA "Hazard Categories" promulgated under Sections 311 and 312 of the Superfund Amendment and Reauthorization Act of 1986 (SARA Title III) and is considered, under applicable definitions, to meet the following categories:

An immediate health hazard

A delayed health hazard

TOXIC SUBSTANCES CONTROL ACT (TSCA): All ingredients are on the TSCA inventory or are not required to be listed on the TSCA inventory.

STATE RIGHT-TO-KNOW:  This product is not known to contain any substances subject to the disclosure requirements of

New Jersey

Pennsylvania

OSHA HAZARD COMMUNICATION STANDARD: This product is a "Hazardous Chemical" as defined by the OSHA Hazard Communication Standard, 29 CFR 1910.1200.

NATIONAL FIRE PROTECTION ASSOCIATION (NFPA) RATINGS:

Health               2

Flammability     1

Reactivity          0

COMPREHENSIVE ENVIRONMENTAL RESPONSE COMPENSATION AND LIABILITY ACT (CERCLA, or SUPERFUND): To the best of our knowledge, this product contains no chemical subject to reporting under CERCLA.

16.	OTHER INFORMATION:

MSDS STATUS: Revised Sections: 3 & 11

Reference: DR-0365-7623

Replaces MSDS Dated: 2/26/03

The Information Herein Is Given In Good Faith, But No Warranty, Express or Implied, Is Made. Consult Dow AgroSciences for Further Information.